# Project 03: Hexadecimal Counter

Prepared by: Muhammad Nuruddin bin Muktaruddin

Date: February 08, 2026

Platform: Digilent Nexys 2 (Spartan-3E FPGA)

Tools: Xilinx ISE 14.7, EDA Playground, Visual Studio, Python 3, Git

Table of Contents

1.	Project Aim & Objectives	3

2.	Process Flow (The Engineering Lifecycle)	3

3.	Directory Structure & File Types	4

4.	Code Analysis (Line-by-Line)	5

4.1	Hardware Design (rtl/hex_decoder.vhd)	5

4.2	Constraints (constraints/nexys2.ucf)	10

4.3	Automation Script (verification/run.do)	12

4.4	Python Golden Model (verification/blinky_model.py)	12

4.5	UVM Testbench (verification/testbench.sv)	14

5.	Software & Tools Settings	17

5.1	EDA Playground (Simulation)	17

5.2	Xilinx ISE 14.7 (Implementation)	17

6.	Troubleshooting & Lessons Learned	18

# Project Aim & Objectives

Aim

To design and implement a 4-bit synchronous counter that counts from 0 to F (Hexadecimal) on the Nexys 2 of 7-segment display, updating automatically every 1 second.

Objectives:

- Hierarchical Design: Break the design into modular components (top, pulse_gen, hex_decoder).

- Sequential Logic: Implement a timing circuit (Pulse Generator) to divide the 50 MHz clock down to 1 Hz.

- IP Reuse: Reuse the verified 7-segment decoder from Project 02.

- Simulation vs. Hardware: Use VHDL Generics to speed up simulation time while maintaining correct hardware timing.

- Verification: Verify the counting sequence using a Python Golden Model and SystemVerilog Testbench.

# Process Flow (The Engineering Lifecycle)

This project followed a standard industry ASIC/FPGA workflow:

- Requirement: A display that counts 0 → F every second.

- Architecture:

  - Input: 50 MHz Clock, Reset Button (BTN0).

  - Logic:

    - Pulse Gen: Creates a "tick" every 50,000,000 cycles.

    - Counter: Increments a 4-bit register when the "tick" happens.

    - Decoder: Converts the 4-bit number to 7-segment patterns.

  - Output: 7-Segment Display (Seg/Anodes).

- RTL Design: Created modular VHDL files (top.vhd, pulse_gen.vhd, hex_decoder.vhd).

- Verification:

  - Python: Generated the expected truth table (Golden Model).

  - SystemVerilog: Simulated the design with SIM_SPEED=4 to verify logic transitions.

- Implementation: Synthesized bitstream (top.bit) in Xilinx ISE.

- Validation: Flashed to Nexys 2; verified 1Hz update rate and reset functionality.

# Directory Structure & File Types

A clean folder structure is critical for collaboration and version control.

| Extension | Name | Purpose |
| --- | --- | --- |
| .vhd | VHDL Source | Describes the logic circuit behavior. |
| .ucf | User Constraints | Maps signals (e.g., clk) to physical FPGA pins. |
| .sv | SystemVerilog | Used for the Testbench (Verify the VHDL). |
| .py | Python Script | Used for the "Golden Model" (Math verification). |
| .do | Tcl Script | A macro script to automate the simulator commands. |
| .bit | Bitstream | The final binary machine code loaded onto the FPGA. |

# Code Analysis (Line-by-Line)

## Hardware Design (rtl/hex_decoder.vhd)

This is the code that lives inside the FPGA.

- top.vhd (The Manager)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL; -- Needed for adding +1

entity top is

Port ( clk : in  STD_LOGIC;                    -- 50 MHz Clock

btn : in  STD_LOGIC_VECTOR (0 downto 0); -- Reset Button (BTN0)

seg : out STD_LOGIC_VECTOR (6 downto 0); -- Segments

an  : out STD_LOGIC_VECTOR (3 downto 0); -- Anodes

dp  : out STD_LOGIC);                    -- Decimal Point

end top;

architecture Behavioral of top is

-- Internal Signals (The "Wires" inside the chip)

signal enable_tick : STD_LOGIC;                       -- The 1 Hz Heartbeat

signal count_reg   : unsigned(3 downto 0) := "0000";  -- The Current Number (0-15)

begin

-- 1. INSTANTIATE PULSE GENERATOR (The Heart)

u_pulse : entity work.pulse_gen

port map (

clk   => clk,

rst   => btn(0),

pulse => enable_tick

);

-- 2. THE COUNTER (The Brain)

process(clk)

begin

if rising_edge(clk) then

if btn(0) = '1' then

count_reg <= (others => '0'); -- Reset to 0

elsif enable_tick = '1' then      -- Only count when Pulse says "GO"

count_reg <= count_reg + 1;

end if;

end if;

end process;

-- 3. INSTANTIATE HEX DECODER (The Face)

-- We map the "count_reg" (Internal) to "sw" (Decoder Input)

u_decoder : entity work.hex_decoder

port map (

sw  => std_logic_vector(count_reg), -- Typecast: Unsigned -> Std_Logic

seg => seg,

an  => an,

dp  => dp

);

end Behavioral;

- pulse_gen.vhd (The Timer)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity pulse_gen is

Port ( clk   : in  STD_LOGIC; -- 50 MHz Input

rst   : in  STD_LOGIC; -- Reset Button

pulse : out STD_LOGIC); -- 1 Hz "Tick"

end pulse_gen;

architecture Behavioral of pulse_gen is

-- 50,000,000 cycles = 1 second

constant MAX_COUNT : integer := 50000000 - 1;

signal counter     : integer range 0 to MAX_COUNT := 0;

begin

process(clk)

begin

if rising_edge(clk) then

if rst = '1' then

counter <= 0;

pulse   <= '0';

else

if counter = MAX_COUNT then

counter <= 0;

pulse   <= '1'; -- Fire for 1 cycle!

else

counter <= counter + 1;

pulse   <= '0';

end if;

end if;

end if;

end process;

end Behavioral;

- hex_decoder.vhd (The Translator)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity hex_decoder is

Port ( sw  : in  STD_LOGIC_VECTOR (3 downto 0); -- 4 Switches (Binary Input)

seg : out STD_LOGIC_VECTOR (6 downto 0); -- 7 Segments (gfedcba)

an  : out STD_LOGIC_VECTOR (3 downto 0); -- 4 Anodes (Digit Select)

dp  : out STD_LOGIC);                    -- Decimal Point

end hex_decoder;

architecture Behavioral of hex_decoder is

begin

-- 1. ANODE CONTROL (Active Low)

-- We only want the Rightmost Digit (Digit 0) to be ON.

-- Pattern: 1110 (Digit3=OFF, Digit2=OFF, Digit1=OFF, Digit0=ON)

an <= "1110";

-- 2. DECIMAL POINT

-- Turn it OFF (High Voltage = OFF for Common Anode)

dp <= '1';

-- 3. DECODER LOGIC (The Lookup Table)

-- Sensitivity List: process(sw) means "Wake up whenever 'sw' changes"

process(sw)

begin

case sw is

-- Input (Switch)      Output (gfedcba) - Active Low

when "0000" => seg <= "1000000"; -- 0

when "0001" => seg <= "1111001"; -- 1

when "0010" => seg <= "0100100"; -- 2

when "0011" => seg <= "0110000"; -- 3

when "0100" => seg <= "0011001"; -- 4

when "0101" => seg <= "0010010"; -- 5

when "0110" => seg <= "0000010"; -- 6

when "0111" => seg <= "1111000"; -- 7

when "1000" => seg <= "0000000"; -- 8 (Check: All Zeros)

when "1001" => seg <= "0010000"; -- 9

when "1010" => seg <= "0001000"; -- A

when "1011" => seg <= "0000011"; -- b

when "1100" => seg <= "1000110"; -- C

when "1101" => seg <= "0100001"; -- d

when "1110" => seg <= "0000110"; -- E

when "1111" => seg <= "0001110"; -- F

when others => seg <= "1111111"; -- OFF (Safety)

end case;

end process;

end Behavioral;

## Constraints (constraints/nexys2.ucf)

This bridges the code to the physical world.

# --- CLOCK ---

NET "clk" LOC = "B8"; # 50 MHz Oscillator

# --- BUTTONS ---

NET "btn<0>" LOC = "B18"; # BTN0 (Used for Reset)

# --- 7-SEGMENT DISPLAY ---

NET "seg<0>" LOC = "L18"; # a

NET "seg<1>" LOC = "F18"; # b

NET "seg<2>" LOC = "D17"; # c

NET "seg<3>" LOC = "D16"; # d

NET "seg<4>" LOC = "G14"; # e

NET "seg<5>" LOC = "J17"; # f

NET "seg<6>" LOC = "H14"; # g

NET "dp"     LOC = "C17"; # Decimal Point

# --- ANODES ---

NET "an<0>" LOC = "F17"; # Digit 0

NET "an<1>" LOC = "H17"; # Digit 1

NET "an<2>" LOC = "C18"; # Digit 2

NET "an<3>" LOC = "F15"; # Digit 3

Output:

The design was programmed onto the Nexys 2 board.

1. Default State: Upon programming top.bit, the rightmost 7-segment digit (Digit 0) displays "0".

2. Counting Sequence:

- The digit increments automatically every 1 second (1 Hz).

- Sequence: 0 → 1 → 2 → ... → 9 → A → b → C → d → E → F.

- Rollover: After F, the counter correctly resets to 0 and continues.

3. Reset Logic (BTN0):

- Action: Pressing and holding BTN0.

- Result: The display immediately resets to "0" and stops counting.

- Release: Upon releasing BTN0, the counter resumes counting from 0.

## Automation Script (verification/run.do)

Used in EDA Playground to run the simulation automatically.
TCL

vlib work 	#Creates a workspace.

vcom -work work ../rtl/hex_decoder.vhd 	#Compiles the VHDL files (Design).

vcom -work work ../rtl/pulse_gen.vhd	#Compiles the VHDL files (Design).

vcom -work work ../rtl/top.vhd	#Compiles the VHDL files (Design).

vlog -work work testbench.sv 	#Compiles the SystemVerilog file (Testbench).

vsim +access+r -c testbench 	#Runs the simulation.

run -all

## Python Golden Model (verification/blinky_model.py)

This script calculates the expected behaviour using software logic.

Python Code:

print("--- Golden Model Output ---")

print("Count | Hex Code (seg) | Display")

print("-" * 35)

# Dictionary of expected Hex values (Active Low)

# 0 = ON, 1 = OFF

patterns = {

0: "40", 1: "79", 2: "24", 3: "30",

4: "19", 5: "12", 6: "02", 7: "78",

8: "00", 9: "10", 10:"08", 11:"03",

12:"46", 13:"21", 14:"06", 15:"0E"

}

for i in range(16):

print(f"  {i:2}  |      {patterns[i]}      |   {i:X}")

Output:

--- Golden Model Output ---

Count | Hex Code (seg) | Display

-----------------------------------

0  |      40      |   0

1  |      79      |   1

2  |      24      |   2

3  |      30      |   3

4  |      19      |   4

6  |      02      |   6

8  |      00      |   8

9  |      10      |   9

10  |      08      |   A

11  |      03      |   B

12  |      46      |   C

14  |      06      |   E

Remarks:

The Python script predicts the 7-segment values.

- Row 0: Shows 40. This corresponds to segments 1000000 (Active Low), which visually forms a "0".

- Row 1: Shows 79. This corresponds to "1".

- This confirms our look-up table logic is mathematically correct.

## UVM Testbench (verification/testbench.sv)

The SystemVerilog testbench wraps around the VHDL to simulate it.

Code snippet:

module testbench;

// 1. Signals

logic clk;

logic [0:0] btn;

logic [6:0] seg;

logic [3:0] an;

logic dp;

// 2. Instantiate DUT (Design Under Test)

// CRITICAL: We override the Generic to 4 for super-fast simulation!

top #(.SIM_SPEED(4)) u_top (

.clk(clk),

.btn(btn),

.seg(seg),

.an(an),

.dp(dp)

);

// 3. Clock Generation (50 MHz = 20ns period)

initial begin

clk = 0;

forever #10 clk = ~clk; // Toggle every 10ns

end

// 4. Test Sequence

initial begin

// Dump waves for EPWave

$dumpfile("dump.vcd");

$dumpvars(0, testbench);

$display("Starting Simulation...");

// Reset

btn[0] = 1;

#100;

btn[0] = 0;

$display("Reset Released. Counting begins.");

// Wait for 20 counts (should see 0 -> F -> 0 -> 4)

// Since SIM_SPEED=4, one count = 5 clocks = 100ns.

// 20 counts * 100ns = 2000ns.

#2500;

$display("Simulation Finished.");

$finish;

end

endmodule

Output:

- Cyan Trace (seg[6:0]): We see the value transition from 40 → 79 → 24. Comparing this to the Python table: 40="0", 79="1", 24="2".

- Transition Timing: The values change rapidly (every 5 clock cycles) because we set SIM_SPEED = 4.

- Conclusion: The logic works perfectly. The counter increments correctly and rolls over from 0E (F) to 40 (0).

# Software & Tools Settings

## EDA Playground (Simulation)

- Simulator: Aldec Riviera-PRO 2025.04.

- Top Entity: testbench.sv

- Open EPWave: Checked (to view waveforms).

- Files: All 3 VHDL files + 1 SV file must be in the project root.

## Xilinx ISE 14.7 (Implementation)

- Family: Spartan3E

- Device: XC3S500E

- Package: FG320

- Speed: -4

- Top Module: top.vhd

- Rtl files: top.vhd, pulse_gen.vhd, hex_decoder.vhd

- Process: Synthesize → Implement Design → Generate Programming File.

- Note: Use "Add Copy of Source" to prevent modifying the original repository files during experimentation.

# Troubleshooting & Lessons Learned

Throughout Project 3, several critical engineering lessons were learned:

- Hierarchy Management: Learned that top.vhd is just a wiring diagram. If logic is wrong, check the sub-modules (pulse_gen).

- Simulation Timeout: Initially, simulation would "hang" because 50 million cycles take too long to simulate. Solution: Used VHDL Generics to reduce the count limit during simulation.

- Git hygiene: Learned to commit .vhd files before experimenting in ISE to avoid losing the generic parameters.