# Project 01: Switch-Controlled

# LED Blinky

Prepared by: Muhammad Nuruddin bin Muktaruddin

Date: January 29, 2026

Platform: Digilent Nexys 2 (Spartan-3E FPGA)

Tools: Xilinx ISE 14.7, EDA Playground, Visual Studio, Python 3, Git

Table of Contents

1.	Project Aim & Objectives	3

2.	Process Flow (The Engineering Lifecycle)	3

3.	Directory Structure & File Types	4

4.	Code Analysis (Line-by-Line)	4

4.1	Hardware Design (rtl/blinky.vhd)	4

4.2	Constraints (constraints/nexys2.ucf)	5

4.3	Automation Script (verification/run.do)	5

4.4	Python Golden Model (verification/blinky_model.py)	6

4.5	UVM Testbench (verification/testbench.sv)	7

5.	Software & Tools Settings	9

5.1	EDA Playground (Simulation)	9

5.2	Xilinx ISE 14.7 (Implementation)	9

6.	Troubleshooting & Lessons Learned	10

# Project Aim & Objectives

The primary goal of this project was to establish a complete RTL-to-Bitstream workflow for FPGA development. It moves beyond a simple "Hello World" by introducing conditional logic based on physical user input.

Specific Objectives:

- Hardware Control: Toggle an LED on/off using a physical slide switch.

- Verification Strategy: Use a Python Golden Model to verify logic before implementation.

- Simulation: Use UVM (Universal Verification Methodology) in EDA Playground to visualize timing.

- Version Control: Maintain a clean, professional Git repository, separating source code from build artifacts.

# Process Flow (The Engineering Lifecycle)

The project followed a strict industry-standard lifecycle to ensure reliability.

- Requirement Analysis: Defined the inputs (Switch 0) and outputs (LED 0).

- Golden Model (Python): Wrote a Python script to calculate the expected "truth."

- RTL Design (VHDL): Wrote the hardware description code.

- Verification (EDA Playground): Simulated the VHDL against the Python model to catch logic errors.

- Synthesis (Xilinx ISE): Compiled the VHDL into physical gates for the Spartan-3E chip.

- Pin Planning (UCF): Mapped abstract VHDL signals to physical pins on the Nexys 2 board.

- Implementation: Generated the .bit file and programmed the board.

# Directory Structure & File Types

A clean folder structure is critical for collaboration and version control.

| Extension | Name | Purpose |
| --- | --- | --- |
| .vhd | VHDL Source | Describes the logic circuit behavior. |
| .ucf | User Constraints | Maps signals (e.g., clk) to physical FPGA pins. |
| .sv | SystemVerilog | Used for the Testbench (Verify the VHDL). |
| .py | Python Script | Used for the "Golden Model" (Math verification). |
| .do | Tcl Script | A macro script to automate the simulator commands. |
| .bit | Bitstream | The final binary machine code loaded onto the FPGA. |

# Code Analysis (Line-by-Line)

## Hardware Design (rtl/blinky.vhd)

This is the code that lives inside the FPGA.

VHDL

library IEEE;

use IEEE.STD_LOGIC_1164.ALL; -- Standard signal definitions

use IEEE.NUMERIC_STD.ALL;    -- Allows math operations (unsigned)

entity blinky is

Port ( clk : in  STD_LOGIC;                      -- 50MHz Clock Input

sw  : in  STD_LOGIC_VECTOR (0 downto 0);  -- Switch Input

led : out STD_LOGIC_VECTOR (0 downto 0)); -- LED Output

end blinky;

architecture Behavioral of blinky is

signal counter : unsigned(25 downto 0) := (others => '0');

begin

process(clk)

begin

if rising_edge(clk) then

counter <= counter + 1; -- Count up every clock tick

-- CONDITIONAL LOGIC

if sw(0) = '1' then

led(0) <= counter(25); -- Blink (Bit 25)

else

led(0) <= '0';         -- Force OFF

end if;

end if;

end process;

end Behavioral;

## Constraints (constraints/nexys2.ucf)

This bridges the code to the physical world.

Plaintext

NET "clk" LOC = "B8";     # Pin B8 -> 50MHz Crystal Oscillator

NET "sw<0>" LOC = "G18";  # Pin G18 -> Switch 0

NET "led<0>" LOC = "J14"; # Pin J14 -> LED 0 (Yellow)

Output:

The design was programmed onto the Nexys 2 board (Spartan-3E). The physical validation confirmed that Switch 0 (SW0) correctly acts as the "Enable" signal for the blinking logic.

| Switch Input (SW0) | Logic State | Expected Output (LED0) | Observed Behaviour | Status |
| --- | --- | --- | --- | --- |
| LOW (0) | Disabled | Forced OFF (Logic 0) | LED remained completely dark. | PASS |
| HIGH (1) | Enabled | Blinking (~0.74 Hz) | LED toggled ON/OFF continuously with a visible period of approx. 1.3 seconds. | PASS |

Analysis of Timing:

The hardware utilizes Bit 25 of the binary counter driven by the 50 MHz onboard oscillator.

- Calculation: F_blink = 50,000,000/ (2^26) = approx. 0.745 Hz

- Observation: The LED blink rate matched the calculated frequency (slow, human-visible toggling), confirming the clock division logic is functioning correctly on the physical silicon.

## Automation Script (verification/run.do)

Used in EDA Playground to run the simulation automatically.

Tcl

vlib work                  # Create "work" library

vcom ../rtl/blinky.vhd     # Compile VHDL Source

vlog +incdir+... testbench.sv # Compile Testbench

vsim -c top -L uvm_1_2     # Run Simulator (Command Line)

run -all                   # Run until finished

## Python Golden Model (verification/blinky_model.py)

This script calculates the expected behaviour using software logic.

Python Code:

class BlinkyFPGA:

def __init__(self, simulation_mode=False):

# State Variables (Registers)

self.counter = 0

self.sw = 0

self.led = 0

# PARAMETERIZATION:

# Hardware uses Bit 25 (Slow). Simulation uses Bit 4 (Fast).

if simulation_mode:

self.target_bit = 4

else:

self.target_bit = 25

def rising_edge_clk(self):

"""Emulates the behavior of the VHDL process(clk)"""

# 1. Counter Logic: Increment by 1

self.counter += 1

# 2. Rollover Logic: Simulate 26-bit hardware limit

if self.counter >= 67108864:

self.counter = 0

# 3. Switch Logic (The core verification target)

if self.sw == 1:

# If Switch is ON, LED takes value of the Target Bit

# (Right shift by target_bit and mask with 1)

self.led = (self.counter >> self.target_bit) & 1

else:

# If Switch is OFF, LED is forced to 0

self.led = 0

# --- MAIN EXECUTION BLOCK ---

if __name__ == "__main__":

# Create instance in Simulation Mode (Fast)

dut = BlinkyFPGA(simulation_mode=True)

dut.sw = 1 # Force switch ON

# Run loop to print cycle-by-cycle behavior

for i in range(35):

dut.rising_edge_clk()

print(f"{i:<10} | {dut.counter:<10} | {dut.led}")

Output:

>>> MODE: Simulation Speedup (Bit 4)

Cycle      | Counter    | LED

------------------------------

0          | 1          | 0

1          | 2          | 0

2          | 3          | 0

3          | 4          | 0

4          | 5          | 0

5          | 6          | 0

6          | 7          | 0

7          | 8          | 0

8          | 9          | 0

9          | 10         | 0

10         | 11         | 0

11         | 12         | 0

12         | 13         | 0

13         | 14         | 0

14         | 15         | 0

15         | 16         | 1 <-- LED Turns ON (Bit 4 High)

16         | 17         | 1

17         | 18         | 1

18         | 19         | 1

19         | 20         | 1

20         | 21         | 1

21         | 22         | 1

22         | 23         | 1

23         | 24         | 1

24         | 25         | 1

25         | 26         | 1

26         | 27         | 1

27         | 28         | 1

28         | 29         | 1

29         | 30         | 1

30         | 31         | 1

31         | 32         | 0 <-- LED Turns OFF (Bit 4 Low)

32         | 33         | 0

33         | 34         | 0

34         | 35         | 0

## UVM Testbench (verification/testbench.sv)

The SystemVerilog testbench wraps around the VHDL to simulate it.

Code snippet:

// 1. INTERFACE: Defines the wires connecting Testbench to DUT

interface blinky_if;

logic clk;

logic [0:0] sw;

logic [0:0] led;

endinterface

// 2. DRIVER: The "Hand" that flips switches

class blinky_driver extends uvm_driver;

// ... (UVM registration code) ...

virtual task run_phase(uvm_phase phase);

// TEST SCENARIO

vif.sw = 0;   // Start with Switch OFF

#2000;        // Wait 2000 nanoseconds

vif.sw = 1;   // Turn Switch ON

#5000;        // Wait 5000 nanoseconds

endtask

endclass

// 3. TOP MODULE: The physical connection

module top;

// Generate Clock: Toggle every 10ns (50 MHz)

logic clk;

initial begin

clk = 0;

forever #10 clk = ~clk;

end

// Instantiate Interface

blinky_if vif();

assign vif.clk = clk;

// Instantiate DUT (Device Under Test) - Connecting VHDL to SV

blinky dut (

.clk(vif.clk),

.sw(vif.sw),

.led(vif.led)

);

// Start UVM Test

initial begin

uvm_config_db#(virtual blinky_if)::set(null, "*", "vif", vif);

run_test("blinky_test");

end

endmodule

Output:

# Software & Tools Settings

## EDA Playground (Simulation)

- Limitation: Cloud simulators are slow. Simulating 1 second of real time (50 million cycles) takes hours.

- Workaround: We implemented a "Simulation Speedup" parameter.

  - Hardware Mode: Uses Bit 25 (Slow blink).

  - Simulation Mode: Uses Bit 4 (Fast blink, only 16 cycles).

- Tools Used: Aldec Riviera-PRO (simulator) and UVM 1.2 (verification library).

## Xilinx ISE 14.7 (Implementation)

- Role: Synthesizes the VHDL into hardware gates.

- Setting: Configured for Spartan-3E (XC3S500E-4FG320).

- Crucial Step: Manually adding the source files from the clean 01_Blinky directory instead of creating them inside the project folder.

# Troubleshooting & Lessons Learned

Throughout Project 1, several critical engineering lessons were learned:

- The "Ghost Folder" Issue:

  - Mistake: Created nested folders like 00_Logbook/00_Logbook.

  - Fix: Flattened the directory structure. Always check file paths before committing.

- Git "Trash" Files:

  - Mistake: The repository was initially cluttered with .ise, .ncd, and .html files.

  - Fix: Created a .gitignore file to automatically exclude build artifacts.

- Naming Mismatch (Critical):

  - Mistake: The UCF file used old names (clk_50mhz, led_out) while the new VHDL used standard names (clk, led). This caused synthesis failure.

  - Fix: Updated UCF to match VHDL port names exactly.

- Python Path Issue:

  - Mistake: Windows "App Execution Alias" prevented Python from running in VS Code.

  - Fix: Used the py launcher or installed Python directly from the Microsoft Store.