# Project 02: Hexadecimal Decoder

Prepared by: Muhammad Nuruddin bin Muktaruddin

Date: February 05, 2026

Platform: Digilent Nexys 2 (Spartan-3E FPGA)

Tools: Xilinx ISE 14.7, EDA Playground, Visual Studio, Python 3, Git

Table of Contents

1.	Project Aim & Objectives	3

2.	Process Flow (The Engineering Lifecycle)	3

3.	Directory Structure & File Types	4

4.	Code Analysis (Line-by-Line)	5

4.1	Hardware Design (rtl/hex_decoder.vhd)	5

4.2	Constraints (constraints/nexys2.ucf)	6

4.3	Automation Script (verification/run.do)	8

4.4	Python Golden Model (verification/blinky_model.py)	8

4.5	UVM Testbench (verification/testbench.sv)	11

5.	Software & Tools Settings	13

5.1	EDA Playground (Simulation)	13

5.2	Xilinx ISE 14.7 (Implementation)	13

6.	Troubleshooting & Lessons Learned	14

# Project Aim & Objectives

Aim

To design and implement a Combinational Logic Circuit on an FPGA that decodes a 4-bit binary input (from physical slide switches) into a human-readable Hexadecimal character (0-9, A-F) on a 7-Segment Display.

Objectives:

- Logic Design: Implement a 4-to-7 binary decoder using VHDL case statements (Look-up Table approach).

- Hardware Control: Master the Common Anode driving technique (Active Low Logic: 0 = ON, 1 = OFF) required for the Nexys 2 display.

- Verification: Develop a "Golden Model" in Python to mathematically verify the segment patterns before implementation.

- Simulation: Validate the logic using a directed SystemVerilog testbench in a cloud-based simulator (EDA Playground).

- Implementation: Synthesize the design, apply pin constraints, and generate a bitstream using Xilinx ISE 14.7.

# Process Flow (The Engineering Lifecycle)

This project followed a standard industry ASIC/FPGA workflow:

- Requirement Analysis: Defined the input (4-bit binary) and output (7-bit "gfedcba" pattern + Anode control).

- Golden Model Generation (Python): Wrote a Python script to generate the Truth Table. This confirmed that displaying "8" requires 0000000.

- RTL Design (VHDL): Translated the Truth Table into a VHDL case statement.

- Verification (SystemVerilog): Created a testbench to drive inputs 0x0 to 0xF and verified the output waveform.

- Constraints Definition (UCF): Mapped VHDL signals to physical pins on the Spartan-3E (e.g., sw<0> to Pin G18).

- Synthesis & Implementation (Xilinx ISE): Generated the .bit file.

- Hardware Validation: Flashed the board and manually verified all 16 states (0-F).

# Directory Structure & File Types

A clean folder structure is critical for collaboration and version control.

| Extension | Name | Purpose |
| --- | --- | --- |
| .vhd | VHDL Source | Describes the logic circuit behavior. |
| .ucf | User Constraints | Maps signals (e.g., clk) to physical FPGA pins. |
| .sv | SystemVerilog | Used for the Testbench (Verify the VHDL). |
| .py | Python Script | Used for the "Golden Model" (Math verification). |
| .do | Tcl Script | A macro script to automate the simulator commands. |
| .bit | Bitstream | The final binary machine code loaded onto the FPGA. |

# Code Analysis (Line-by-Line)

## Hardware Design (rtl/hex_decoder.vhd)

This is the code that lives inside the FPGA.

VHDL

library IEEE;

use IEEE.STD_LOGIC_1164.ALL; -- Standard signal definitions

use IEEE.NUMERIC_STD.ALL;    -- Allows math operations (unsigned)

entity hex_decoder is

Port ( sw  : in  STD_LOGIC_VECTOR (3 downto 0); -- 4-bit Binary Input

seg : out STD_LOGIC_VECTOR (6 downto 0); -- 7 Segments (g-a)

an  : out STD_LOGIC_VECTOR (3 downto 0); -- 4 Anodes (Digit Select)

dp  : out STD_LOGIC);                    -- Decimal Point

end hex_decoder;

architecture Behavioral of hex_decoder is

begin

-- 1. ANODE CONTROL: Select only the Rightmost Digit (Digit 0)

-- '0' = ON for Anodes. "1110" turns ON Digit 0, others OFF.

an <= "1110";

dp <= '1'; -- Turn Decimal Point OFF (High Voltage)

-- 2. DECODER LOGIC: Updates instantly when 'sw' changes

process(sw)

begin

case sw is

-- Input      Output (gfedcba) - Active Low

when "0000" => seg <= "1000000"; -- Display 0

when "1000" => seg <= "0000000"; -- Display 8 (All Segments ON)

-- ... (Full table 0-F implemented here)

when others => seg <= "1111111"; -- Default OFF (Safety)

end case;

end process;

end Behavioral;

## Constraints (constraints/nexys2.ucf)

This bridges the code to the physical world.

Plaintext

# INPUTS (Switches)

NET "sw<0>" LOC = "G18"; # LSB

NET "sw<3>" LOC = "K17"; # MSB

# OUTPUTS (7-Segment Display)

NET "seg<0>" LOC = "L18"; # Segment 'a'

NET "seg<6>" LOC = "H14"; # Segment 'g'

NET "dp"     LOC = "C17"; # Decimal Point

# ANODES (Digit Selection)

NET "an<0>" LOC = "F17"; # Digit 0 (Rightmost)

NET "an<3>" LOC = "F15"; # Digit 3 (Leftmost)

Output:
The design was programmed onto the Nexys 2 board. All 16 input combinations of the 4-bit switch vector (sw[3:0]) were tested. The 7-segment display output matched the expected Hexadecimal values exactly, confirming the Active Low logic and pin constraints.

| Binary Input (SW3-SW0) | Decimal Value | Expected Hex | Observed Display | Status |
| --- | --- | --- | --- | --- |
| 0000 (All Low) | 0 | 0 | 0 | PASS |
| 0001 (SW0 High) | 1 | 1 | 1 | PASS |
| 0010 (SW1 High) | 2 | 2 | 2 | PASS |
| 0011 (SW1+0 High) | 3 | 3 | 3 | PASS |
| 0100 (SW2 High) | 4 | 4 | 4 | PASS |
| 0101 (SW2+0 High) | 5 | 5 | 5 | PASS |
| 0110 (SW2+1 High) | 6 | 6 | 6 | PASS |
| 0111 (SW2+1+0 High) | 7 | 7 | 7 | PASS |
| 1000 (SW3 High) | 8 | 8 | 8 | PASS |
| 1001 (SW3+0 High) | 9 | 9 | 9 | PASS |
| 1010 (SW3+1 High) | 10 | A | A | PASS |
| 1011 (SW3+1+0 High) | 11 | B | b (Lowercase)* | PASS |
| 1100 (SW3+2 High) | 12 | C | C | PASS |
| 1101 (SW3+2+0 High) | 13 | D | d (Lowercase)* | PASS |
| 1110 (SW3+2+1 High) | 14 | E | E | PASS |
| 1111 (All High) | 15 | F | F | PASS |

*Note: 'B' and 'D' are displayed in lowercase ('b', 'd') to avoid confusion with '8' and '0'.

## Automation Script (verification/run.do)

Used in EDA Playground to run the simulation automatically.

Tcl

vlib work                  # Create library

vcom hex_decoder.vhd       # Compile VHDL Design

vlog testbench.sv          # Compile SystemVerilog Testbench

vsim +access+r -c top      # Initialize Sim (Enable Read Access for Waveforms)

run -all                   # Run until completion

## Python Golden Model (verification/blinky_model.py)

This script calculates the expected behaviour using software logic.

Python Code:

def get_seven_segment_code(value):

"""

Returns the 7-bit pattern (gfe_dcba) for a given hex digit (0-15).

Logic: 0 = ON, 1 = OFF

"""

# Map: '0' needs a,b,c,d,e,f ON (0). g OFF (1).

# Pattern: g f e d c b a

patterns = {

0:  "1000000", # 0

1:  "1111001", # 1 (only b, c on)

2:  "0100100", # 2

3:  "0110000", # 3

4:  "0011001", # 4

5:  "0010010", # 5

6:  "0000010", # 6

7:  "1111000", # 7

8:  "0000000", # 8 (all on)

9:  "0010000", # 9

10: "0001000", # A

11: "0000011", # b (lower case)

12: "1000110", # C

13: "0100001", # d (lower case)

14: "0000110", # E

15: "0001110"  # F

}

return patterns.get(value, "1111111") # Default OFF

# --- MAIN EXECUTION ---

if __name__ == "__main__":

print(f"{'Hex':<5} | {'Binary (gfedcba)':<18} | {'Active Low Check'}")

print("-" * 45)

for i in range(16):

hex_char = hex(i).upper().replace("0X", "")

pattern = get_seven_segment_code(i)

print(f"{hex_char:<5} | {pattern:<18} | {'Valid'}")

Output:

Hex   | Binary (gfedcba)   | Active Low Check

---------------------------------------------

0        | 1000000                   | Valid

1        | 1111001                   | Valid

2        | 0100100                   | Valid

3        | 0110000                   | Valid

4        | 0011001                   | Valid

5       | 0010010                | Valid

6       | 0000010                | Valid

7       | 1111000                | Valid

8       | 0000000                | Valid

9       | 0010000                | Valid

A       | 0001000                | Valid

B       | 0000011                | Valid

C       | 1000110                | Valid

D       | 0100001                | Valid

E       | 0000110                 | Valid

F       | 0001110                 | Valid

## UVM Testbench (verification/testbench.sv)

The SystemVerilog testbench wraps around the VHDL to simulate it.

Code snippet:

// Testbench for Hex Decoder

module top;

// Signals

logic [3:0] sw;   // 4 Switches

logic [6:0] seg;  // 7 Segments (Output)

logic [3:0] an;   // Anodes (Output)

logic dp;         // Decimal Point (Output)

// Instantiate the VHDL DUT (Device Under Test)

hex_decoder dut (

.sw(sw),

.seg(seg),

.an(an),

.dp(dp)

);

// Test Logic

initial begin

$dumpfile("dump.vcd"); // Create waveform file

$dumpvars;

// Title

$display("-------------------------------------------");

$display("HEX | SW (Input) | SEG (Output) | Check");

$display("-------------------------------------------");

// Loop from 0 to 15 (0x0 to 0xF)

for (int i = 0; i < 16; i++) begin

sw = i;      // Apply Input

#10;         // Wait 10ns for logic to settle

// Print status

// %h = Hex format, %b = Binary format

$display(" %h  |    %b    |   %b    |", sw, sw, seg);

end

$display("-------------------------------------------");

$finish; // End simulation

end

endmodule

Output:

# Software & Tools Settings

## EDA Playground (Simulation)

- Simulator: Aldec Riviera-PRO 2023.04 (or later).

- Testbench Language: SystemVerilog / Verilog.

- Design Language: VHDL.

- Run Options: Checked "Use run.do Tcl file".

- Waveform Viewer: EPWave (Enabled via +access+r flag).

## Xilinx ISE 14.7 (Implementation)

- Family: Spartan3E

- Device: XC3S500E

- Package: FG320

- Speed: -4

- Top Module: hex_decoder

- Process: Synthesize → Implement Design → Generate Programming File.

# Troubleshooting & Lessons Learned

Throughout Project 1, several critical engineering lessons were learned:

- Waveform "Read Access" Error:

- Issue: Initially, EPWave would not load, and the console reported KERNEL_0085 "sw" does not have read access.

- Solution: Added the +access+r flag to the vsim command in run.do. This forces the simulator to expose signals for waveform viewing.

- Common Anode Logic:

- Concept: The Nexys 2 display is Common Anode, meaning the FPGA must sink current (Output 0) to turn a segment ON.

- Verification: The Python Golden Model proved that "8" corresponds to 0000000 (All Zeros), preventing logic inversion errors in VHDL.

- Digit Ghosting:

- Concept: Since all 4 digits share the same segment lines, enabling all Anodes (an <= "0000") would show the same number on all 4 digits.

- Implementation: Fixed an <= "1110" to ensure only the rightmost digit displays the value.